4.ESP32 for Web Interface, Stereo Vision Handling, and Communication Hub

For this esp32 second module as per suggestion add Web Interface, Stereo Vision Handling, and Communication Hub .

Code name :- esp32_sketch_WebUI_and_Master_communication

Explanation:

- Web Interface: Hosts a responsive manual control portal with stereo video stream placeholders and robotic arm visualization using SVG.

- Stereo Vision Handling: Shows streams from stereo camera ESP32-CAM modules via network-accessible MJPEG streams embedded in the UI.

- Communication Hub: Sends user commands received from the web UI over UART to the navigation ESP32 (your first module), enabling coordination of autonomous navigation and arm control.

- WiFiMulti: Ensures robust Wi-Fi connectivity by managing multiple networks.

- Real-time Arm Visualization: Joint angles from sliders dynamically update an SVG drawing representing the robotic arm pose.

This code forms the central hub of user interfacing, stereo video monitoring, and inter-controller communication suitable for your project architecture.

If you'd like, I can also help with sample code for the navigation ESP32 module, stereo depth handling, or advanced communication protocols next.

Code after slave master program addition

Explanation:

- The code manages multiple Wi-Fi networks with WiFiMulti for robust connectivity.

- A web server serves a rich control page showing live stereo camera streams, robotic arm joint angle controls, veggie picking inputs, and an SVG arm visualization.

- User inputs (joint angles, pick commands) send HTTP GET requests to /command, which forwards commands via UART to the slave ESP32.

- Communication with the slave ESP32 uses UART2 on GPIO16 (RX) and GPIO17 (TX).

- The design separates web serving and control UI (master ESP32) from sensor reading and actuation (slave ESP32).

This combined code meets your architecture for a master ESP32 handling responsive web UI and streaming, with UART command forwarding to a capable slave ESP32 managing robot core functions.

Here is the rewritten master ESP32 code that integrates:

- Multi-network Wi-Fi connection management

- Web server serving control UI with stereo camera streams, arm controls, and an added real-time robot pose and obstacle visualization canvas

- UART2 communication with slave ESP32 using binary protocol with parsing and sending

- JSON /status endpoint serving latest parsed slave status for UI visualization

- Command forwarding from web UI to slave ESP32

Explanation:

- Maintains your original control UI and stereo video streams.

- Adds a real-time canvas showing robot’s estimated position, heading as arrow, and obstacle range.

- Receives binary packets from slave ESP32 over UART; updates position and sensor values.

- Exposes /status HTTP endpoint returning JSON for UI canvas updates.

- Sends control commands from UI sliders and buttons to slave ESP32 UART.

- Uses binary framed UART protocol for reliability and reduced latency.

- Multi-network Wi-Fi auto-connect handled by WiFiMulti.

This complete code ties together robust communication, rich UI, and your robot’s sensor-driven feedback for effective monitoring/control.

If you want further help integrating more SLAM visualization or AI features, just ask!